Don't _ever_ run without a thermostat , it's very bad for the engine

as the coolant circulates too rapidly to pick up heat so the engine

is hotter that the coolant is . I use a 187° F 'Stat , you can use a

160° one if you're worried about overheating but the closer you can

run it to boiling temperature , the longer your engine will last ,

make more power and better fuel mileage too .



_You_ can clean out that funky old radiator and the water jacket

too  @ home and easily , here's how :



Get a couple pounds of Citric Acid Powder (bakery & candy supply

stores or Flea-Bay) , mix one pound of the Citric Acid into three

gallons of warm or hot water , stir until dissolved .

NOTE : I use and recommend using distilled water here....



Drain your cooling system and remove the block drain petcock , flush

with a hose until clear water comes out , close it up and re - fill ,

ad 1/2 to one cup of _non_sudsing_ detergent (SHOUT etc.) and crank

it up , run it until it reaches operating temperature , driving not

just idling , then drain that off and open 'er up , flush with the

garden hose again until clear water comes out every where .



Close it back up and fill with the Citric Acid / water mix , run it

for at LEAST 45 minutes , I like to drive it to work and back a few

times .



The coolant will turn a thick red or even black , this is the rusty

crud you want out .



Drain and flush , re-fill with plain (distilled) water , run engine
until it reaches operating temperature (it'll turn red again,
sometimes it turns black)

drain and flush , repeat the fill , drain & flush until the water stays

clear , this may take a while as the water jacket is usually full of

very fine silty rust that doesn't come loose until the engine is

close to boiling point .



All clean now ? good ~ drain flush one last time and re - fill with a

coolant and water mix , using 40 % coolant & 60 % distilled water .



I use and highly recommend , distilled water for all my cooling

system needs including flushing & cleaning , it's usually .99 CENTS

at the drugstore if you check all of them .



Try this first as if the radiator is clogged / dirty , so is the

water jacket and it'll just clog up your spiffy radiator if you send

it out then install it again ....



_YOU_ will do a better job than any shop will because it's a time

consuming , labor intensive job , if rediculously simple and easy...
it just takes time to do .


BTW : you'll know it's really & truly clean when you can drive it

for an hour then wipe inside the radiator with your finger and -not-

have it come out red....

Citric Acid Powder will dissolve rust & scale , hard water deposits

and radiator solder bloom , this is the white crusty looking stuff you

can see on the ends of the tubes when you peer inside the filler

neck .



It cannot hurt plastic , brass nor aluminum so it's good for that old '79 Honda

Civic out back too .



You _MUST_ first de-grease the cooling system or it won't work .



Lastly , I find the lower tank in the radiator usually has some

heavier crud/chunks in it after all else is clean , I always remove
the

radiator and flush this last bit out with the hose and shaking ,

holding it " Just So" to facilitate the draining out of the crud

you'll easily see in the lower hose connection .



-Nate