## Eaton Single Reduction Single Drive Axles.

Click here to Identify your axle

#### Select a Model (28 Models)

- 15040S,

- 19060S,A,D

- 21060S,A,D

- 21080S,A,D

- 22060S,A,D

- 22080S,A,D

- 23080S,A,D

- 23105S,A,D

- 26105S,A,D

- 30105S,D

#### Available Ratios

| 15040S | 19060S,A,D, 21060S,A,D 22060S,A,D |
| --- | --- |
| 3.36 | 2.64 |
| 3.55 | 2.85 |
| 3.70 | 3.08 |
| 3.90 | 3.25 |
| 4.11 | 3.36 |
| 4.33 | 3.55 |
| 4.63 | 3.70 |
| 4.88 | 3.90 |
| 5.29 | 4.11 |
| 5.38 | 4.33 |
| 5.57 | 4.63 |
| 5.83 | 4.88 |
| 6.17 | 5.29 |
| 6.50 | 5.57 |
| 7.17 | 6.17 |
|  | 6.50 |
|  | 7.17 |

#### Available Ratios

| 21080S,A,D 22080S,A,D 23080S,A,D | 23105S,A,D 26105S,A,D 30105S,D |
| --- | --- |
| 2.85 | 2.85 |
| 3.08 | 3.08 |
| 3.25 | 3.36 |
| 3.36 | 3.58 |
| 3.55 | 3.70 |
| 3.70 | 3.90 |
| 3.90 | 4.11 |
| 4.11 | 4.33 |
| 4.33 | 4.63 |
| 4.63 | 4.88 |
| 4.88 | 5.25 |
| 5.29 | 6.14 |
| 5.57 |  |
| 5.83 |  |
| 6.17 |  |
| 6.50 |  |

MODEL IDENTIFICATION

The Axle Gear Ratio can be located on the identification tag on the carrier assembly or on the axle tube.

Example: 15040S

First & Second Digits = GAWR x 1000 lbs.

Third, Fourth & Fifth Digits = GCW x 1000 lbs.

Sixth Digit S = Single Reduction

Sixth Digit T = 2-Speed

Sixth Digit P = Planetary Double Reduction

Sixth Digit D = Single Reduction with Wheel Differential Lock

Sixth Digit A = Single Reduction with in Axle Speed Sensing

This page was last modified

© Copyright 1994 - 2009 Vibrate Software - All Rights Reserved