1971-1972 K 20 Hub lock assembly

All items are in Group 6.308

1.    Gasket, frt. wheel locking hub retainer                                                            3958034                        2      $8.53 *

2.    Housing, frt. wheel locking hub retainer                                                        3782287                        2

3.    Gasket, frt. wheel locking hub retainer                                                              3979441                      2      $2.13 *

4.    Ring, frt. Wheel locking hub retainer                                                                    3958029                      2    $91.00 **

5.    Collar, frt. Wheel locking hub                                                                                        3782285                      2

6.    Gear, frt. Wheel locking hub                                                                                            3958043                      2

7.    Bolt, frt. Wheel locking hub retainer                                                                    3958044                    12    $16.00 **

8.    Washer, lock retainer                                                                                                                  3979606                    12

9.    Knob, actuating                                                                                                                                    3995867                      2

10. Seal, frt. Wheel locking hub lock actuator knob                              3995885                        2

11. Ring, frt. Wheel locking hub                                                                                          3958038                          2

12. Cam Assy., frt. wheel locking hub lock                                                          3958039                      2    $35.00 **

13. Ring, frt. Wheel locking hub                                                                                          3958043                      2    $4.25*

14. Spring, frt. Wheel locking hub                                                                                                  ???                          2

15. Pin, frt. Wheel locking hub                                                                                                3958040                      2

There are some problems with the GM manual on the names given to parts in the exploded views and in the descriptions for part #’s.

Rings are bad.    Three are shown and three are listed. They are;

70-72 Ser. 20(4/wd) 3958043    2      Ring, lock hub axle shaft gear retaining (Dana 35389) (note 2)

70-72 Ser. 20(4/wd) 3958038    2      Ring, lock retaining (Dana 53583) (note 2)

70-72 Ser. 20(4/wd) 3958029    2      Ring, w/ bushing clutch lock hub inner (Dana26587-X) (note 2)

I determined the numbers by pricing the items.

Note 2 is all used on plastic actuator knobs

And a spring for 71-72 K-20 is not listed.

* dealer available

** found elsewhere