Chevy Truck - Front Suspension Bolt Torque (ft. lbs.)§*

| Model | C-10 | CP-20-30 |
| --- | --- | --- |
| Lower Control Arm Shaft U-bolt | 85 | 85 |
| Upper Control Arm Shaft Nuts | 70 | 105 |
| Control Arm Rubber Bushings | 115 | ----- |
| Upper Control Arm Bushing Steel §§ |  | New 190 Used 115 |
| Lower Control Arm Bushing Steel §§ |  | New 280 Used 115 |
| Upper Ball Joint Nut | * 50 | ** 90 |
| Lower Ball Joint Nut | **90 | **90 |
| Nut, Spindle To Knuckle | ----- | ----- |
| Crossmember To Side Rail Ў | 65 | 65 |
| Crossmember To Bottom Rail ЎЎ | 90 | 90 |
| Crossmember Brake Support Struts | 60 | 60 |
| Stabilizer Bar To Control Arm | 25 | 25 |
| Stabilizer Bar To Frame | 25 | 25 |
| Shock Absorber Upper End | 140 | 140 |
| Shock Absorber Lower End | 60 | 60 |
| Brake Splash Shield To Knuckle | 120 in. lbs. | 120 in. lbs. |
| Wheel Bearing Adjustment | ---- | ---- |
| Wheel Bearing End Movement | .001-.005" | .001-.005" |
| Caliper Mounting Bolt | 35 | 35 |
| Spring - Front Eye Bolt | ---- | ---- |
| Spring - Rear Eye Bolt | ---- | ---- |
| Spring - To Rear Shackle Bolt | ---- | ---- |
| Spring - To Axle U-Bolt | ---- | ---- |
| Spring - Front Support To Frame | ---- | ---- |
| Suspension Bumper | 15 | 15 |
| Stabilizer To Spring Plate | ---- | ---- |

* Plus additional torque to align cotter pin. Not to exceed 90 ft. lbs. maximum.

** Plus additional torque to align cotter pin. Not to exceed 130 ft. lbs. maximum.

*** Plus additional torque to align cotter pin.

Ў P300 (32), P300 (42) and JF-9 - 100 ft. lbs.

ЎЎ P300 (32) - 215 ft. lbs., P300 (42) and JF-9 - 130 ft. lbs.

ЎЎЎ (K10 - 20) 205 ft. lbs., (K30) 65 ft. lbs.

§ All specifications are given in ft. lbs of torque unless otherwise specified.

§§ C10, G10-20 Rubber Bushings; C20 -30, P10-30 Steel Bushings.

For Axle U-Bolt tightening sequence, see below.