1962 CHEVY STEPSIDE PICKUP NOSTALGIA PRO/STREET

| Facts & Figures |  |
| --- | --- |
| 1Bad62Pro/Street |  |
| Cleveland County, NC |  |
| 1962 Chevrolet C10 Pickup 1/2 Ton Stepside Shortbed |  |
| CHASSIS |  |
| Frame: | Custom hybrid - Hand Fabricated - Custom Suspension - Altered Body & Frame |
| Modifications: | 1976 Mustang II IFS (Manual Rack Steering) 58 Ford Wagon 9” Rear End |
| Rear-end / Ratio: | 3.70 Gear Ratio /One Legger for now |
| Rear suspension: | 58 Ford Wagon 9” Rear End Competition Engineering Rear Coil-Over Shock Kit with 250lb springs included Rear Coil-Over Shock Kits with Springs |
| Rear brakes: | Upgraded 11” Drums & Brakes |
| Front suspension: | Mustang II IFS (Manual Rack Steering) Heidts Superirde Front Coil-Over Shocks for Mustang II Suspension |
| Front brakes: | Stock Mustang II brakes |
| Steering box: | Mustang II IFS rack-and-pinion (Manual Rack Steering) |
| Front wheels: | Weld Racing Draglite 90-Series Wheel 15 x 7 |
| Rear wheels: | Weld Racing Draglite 90-Series Wheel 15 x 12 |
| Front tires: | Falken Visa 912 Tires - 758 P215/65R15 96H |
| Rear tires: | Mickey Thompson Sportsman Pro Tire, 29x15.50-15LT |
| Gas tank: | Jegs 20-Gallon Aluminum Fuel Cell |
| DRIVETRAIN |  |
| Engine: | Chevy 1969 C-10 350ci Cast Iron LS9 3970010 4 Bolt Main 350ci 4 Bolt Main (.30 over) Camshaft Hydraulic Dual Pattern (Lift .443” Int., .465” Exh.) Crankshaft Nodular Cast iron |
| Machining: | Local Shop |
| Custom Fabrication | 1Bad62Pro/Street’s Father |
| Heads: | 1968 Chevrolet Corvette 186 – 63cc 327 Double/Camel Hump New Springs – 1969 – Z28 New Stainless Valves Manley 1.94 Int / 1.50Exh Hardened Exh Seats Setup to 490 Lift New Retainers and Locks New Guides |
| Valve covers: | JEGS/ARP/Edelbrock Elite Series Dress-Up Kit Edelbrock Elite Tall Valve Covers / Breathers / Air Cleaner JEGS Steel Core Gaskets ARP Valve Cover Bolts JEGS Valve Cover T-Bar Wing Nuts - Chrome Steel |
| Manifold / Induction: | Edelbrock Performer RPM EnduraShine Finish / Edelbrock Performer 600-cfm Carb. |
| Ignition: | Pertronix Flame Thrower HEI Distributor Yellow 8mm Delco |
| Headers: | Headers Hooker 70 Corvette 22”x 5” Baloneys Chrome Exhaust Tips |
| Exhaust / Mufflers: | Home Made Exhaust Old School Mufflers made by 1Bad62Pro/Street’s father |
| Pulley system: | Short Water Pump System |
| Transmission: | 1970 Chevy TH350 2,200 stall converter B&M Shift Improver Kit Built for Street Cruising |
| Modifications: | Chevy 350ci Engine 10.5:1 Compression Ratio Pistons Hyperedic Cast Aluminum Flat Top Camshaft Hydraulic Dual Pattern (Lift .443” Int., .465” Exh.) Crankshaft Nodular Cast iron |
| Shifter: | Lokar TH350 Auto Trans Shifter Tail-Shaft Mount |
| BODY |  |
| Style: | 1962 Chevrolet C10 Pickup 1/2 Ton Stepside Shortbed |
| Modifications: | 1Bad62Pro/Street’s Father and 1Bad62Pro/Street |
| Fenders front / rear: | Stock, original |
| Hood: | Fiberglass 6" tall cowl induction hood – Needs fixed – Bought it new and it looks terrible! What you see is what was shipped to me. Unlimited Performance Products |
| Grille: | Stock / Anodized Aluminum Grille |
| Bed: | Custom Mounted Fuel Cell, Maybe Wood / Steel / Aluminum Electric Fuel Pump Street 95 GPH Pre-set to approximately 6.5 psi Street Fuel Filter 2.45" diameter x 5.25" high |
| Bodywork and paint by: | TBD |
| Paint type / Color: | Thinking about PPG Lemon Ice Yellow or 2004 Ford Mustang Screaming Yellow with Old School Phat Candy Flames |
| Headlights / Taillights: | Stock - Half moons / Maybe Cadillac tail lights |
| Outside mirrors: | Stock / Peep mirrors with visors |
| Bumpers: | Stock / Modified |
| INTERIOR |  |
| Dashboard: | Stock with some modifications |
| Gauges: | Auto Meter / Monster Tach / Auto gages |
| Air conditioning: | Roll Down the Windows! Grab a sweat rag |
| Stereo System: | Original Delco AM Radio / Exhaust sound of a dual pattern cam |
| Steering wheel: | Grant Silver Basic Classic Foam Steering Wheel |
| Steering column: | 73 Nova / Modified / Going to swap out for tilt later |
| Seat: | Recovered Original Bench / May change to bucket later |
| Upholstery | TBD |
| Material / Color: | TBD |
| Carpet: | Maybe Black |
| Battery: | Optima Dry Cell Battery / Located behind the seat |